Flight Maintenance Instructions

| Frequency | Maintenance Required |
| --- | --- |
| Daily | Clean door seal Clean the external surface Drain used water tank |
| Weekly | Clean distilled water reservoir (drain tank) Clean sterilization chamber |
| Monthly | Clean filter inside the chamber and inside the distilled water tank |
| Every 6/12 Months | Replace door seal |
| Every Year | Replace silicone tubing, filters and door seal |
| Every 2 Years | Replace battery on main PCB that controls timer |

Clean Distilled Water Tank Weekly			Clean Chamber Weekly

Clean the distilled water tank weekly using water	Remove all trays and the tray rack from the

and a soft cloth. Alcohol can also be used to help	chamber. Clean the chamber with a soft cloth

clean and disinfect reservoir. Apply alcohol to a		using only distilled water.

cloth or on a towel only.

Clean the Door Seal Daily

Clean the door seal with a smooth cloth and distilled water.